⚙️ General AVR Compilation and Programming Steps

ATmega328P • AVR-GCC • AVRDUDE

This guide explains the basic workflow for compiling and programming a bare-metal Embedded C program for the ATmega328P using AVR-GCC and AVRDUDE.

## 1️⃣ Write the Source Code

Create and write your bare-metal C program, for example blink.c, using any text editor.

nano blink.c

You can also use other editors such as VS Code or Vim.

## 2️⃣ Compile the C Program to an ELF File

Use avr-gcc to compile the C source code into an ELF (Executable and Linkable Format) file. The -mmcu=atmega328p option specifies the target microcontroller, while -Os optimizes the program for smaller code size.

avr-gcc -mmcu=atmega328p -Os -o blink.elf blink.c

🔍 Command Breakdown

| Option | Purpose |
| --- | --- |
| avr-gcc | AVR C compiler |
| -mmcu=atmega328p | Specifies the target MCU |
| -Os | Optimizes for smaller code size |
| -o blink.elf | Specifies the output file |
| blink.c | Input C source file |

📄 Output: blink.elf

## 3️⃣ Convert ELF to Intel HEX

AVR programmers commonly use Intel HEX files for programming Flash memory. Use avr-objcopy to convert the ELF file into a HEX file.

avr-objcopy -O ihex -R .eeprom blink.elf blink.hex

🔍 Command Breakdown

| Option | Purpose |
| --- | --- |
| avr-objcopy | Converts the file into another format |
| -O ihex | Specifies Intel HEX as the output format |
| -R .eeprom | Excludes the EEPROM section |
| blink.elf | Input ELF file |
| blink.hex | Output HEX file |

📄 Output: blink.hex

## 4️⃣ Check Program Memory Usage

Use avr-size to check how much Flash and SRAM your program uses.

avr-size --format=avr --mcu=atmega328p blink.elf

The ATmega328P provides:

💾 32 KB Flash memory

🧠 2 KB SRAM

🗃️ 1 KB EEPROM

This step helps verify that your program fits within the available memory.

## 5️⃣ Detect the Serial Port

Connect the development board to your computer using USB. On Linux, the board may appear as:

/dev/ttyACM0

/dev/ttyUSB0

To identify the available serial ports:

ls /dev/ttyACM* /dev/ttyUSB*

💡 Tip: If the command does not show anything, check the USB connection and make sure the board is properly detected by Linux.

## 6️⃣ Flash the HEX File to the ATmega328P

Use avrdude to upload the HEX file to the ATmega328P through the board's bootloader.

avrdude -p m328p -c arduino -P /dev/ttyACM0 -b 115200 -U flash:w:blink.hex:i

🔍 Command Breakdown

| Option | Purpose |
| --- | --- |
| avrdude | AVR programming utility |
| -p m328p | Specifies the ATmega328P |
| -c arduino | Uses the Arduino bootloader protocol |
| -P /dev/ttyACM0 | Specifies the serial port |
| -b 115200 | Specifies the baud rate |
| -U | Performs a memory operation |
| flash | Selects Flash memory |
| w | Write operation |
| blink.hex | HEX file to upload |
| i | Intel HEX format |

🚀 After successful programming, the ATmega328P will execute the uploaded program.

## 🔄 Complete Compilation Workflow

📝 blink.c
           │
           │ avr-gcc
           ▼
       📦 blink.elf
           │
           │ avr-objcopy
           ▼
       📄 blink.hex
           │
           │ avrdude
           ▼
    🔧 ATmega328P Flash
           │
           ▼
      ⚡ Program Runs

## 🧰 Tools Used

| Tool | Function |
| --- | --- |
| avr-gcc | 🛠️ Compiles Embedded C code |
| avr-objcopy | 🔄 Converts ELF to HEX |
| avr-size | 📊 Checks memory usage |
| avrdude | 🚀 Programs the ATmega328P |

💡 Note: The serial port, programmer type, and baud rate depend on the board and programming method. The example above assumes an ATmega328P board using an Arduino-style bootloader.